AI-Based Species-Specific Prevention System

| Species | Threat Level | Primary Deterrent | Secondary Deterrent | Why It Works | Limitations |
| --- | --- | --- | --- | --- | --- |
| Elephant | Very High | High-intensity directional LED floodlights (100–200 W) | Forest Department alert, camera tracking, directional speaker with species-specific audio | Sudden bright directional light and targeted sounds can encourage route changes without harming animals | Sirens alone are generally ineffective for habituated elephants. Human intervention may still be required. |
| Wild Boar | High | Directional predator sounds (tiger/leopard) | Flashing LED lights, vibration devices near entry points | Boars are sensitive to unfamiliar sounds and sudden visual stimuli | Animals can habituate if the same pattern is repeated continuously. |
| Monkey | Medium | Monkey distress calls or predator calls | Flashing lights and rotating visual deterrents | Social animals respond to species-specific warning calls | Effectiveness decreases if overused. |
| Nilgai | High | Directional floodlights | Intermittent warning sounds and farmer notification | Nilgai often avoid sudden illumination and repeated disturbance | May require repeated activation if food availability is high. |
| Deer | Medium | Flashing lights | Low-volume warning sounds | Deer are cautious around unexpected light and movement | Limited effect if animals are already habituated. |
| Stray Cattle | Medium | Farmer mobile notification | Localized buzzer or warning light | Allows farmer intervention without disturbing the entire village | Requires manual removal. |
| Snake | Medium | Electromagnetic/Vibration Repellent (experimental) | Farmer notification and camera monitoring | Some commercial systems claim snakes avoid ground vibrations or EM fields | Scientific evidence is limited; effectiveness varies by species and environment. |
| Crab | Low | Electromagnetic/Vibration Repellent (experimental) | Local barrier or farmer notification | May discourage movement in limited areas by altering substrate vibrations | Limited coverage; effectiveness is inconsistent and not suitable as a standalone solution. |

AI-Based Species-Specific Prevention System

| Stage | Action |
| --- | --- |
| Stage 1 | Detect animal using AI (YOLO) |
| Stage 2 | Identify species |
| Stage 3 | Check crop type (cotton, maize, sugarcane, etc.) |
| Stage 4 | Check intrusion location and time |
| Stage 5 | Select the most suitable deterrent combination |
| Stage 6 | Monitor whether the animal leaves |
| Stage 7 | Escalate only if the first response fails |

Example Decision Matrix

| Species | Lights | Directional Speaker | Sprinkler | Farmer Alert | Forest Alert | Camera Tracking |
| --- | --- | --- | --- | --- | --- | --- |
| Elephant | ✓ | ✓ | ✗ | ✓ | ✓ | ✓ |
| Wild Boar | ✓ | ✓ | Optional | ✓ | ✗ | ✓ |
| Monkey | ✓ | ✓ | ✗ | ✓ | ✗ | ✓ |
| Nilgai | ✓ | Optional | ✗ | ✓ | ✗ | ✓ |
| Deer | ✓ | Optional | ✗ | ✓ | ✗ | ✓ |
| Stray Cattle | ✓ | ✗ | Optional (non-cotton only) | ✓ | ✗ | ✓ |

Crop-Aware Decision Engine

| Crop | Sprinkler | Flood Light | Speaker | Farmer Alert |
| --- | --- | --- | --- | --- |
| Cotton | ✗ | ✓ | ✓ | ✓ |
| Wheat | ✓ | ✓ | ✓ | ✓ |
| Rice | ✓ | ✓ | ✓ | ✓ |
| Sugarcane | Optional | ✓ | ✓ | ✓ |
| Vegetables | ✓ | ✓ | ✓ | ✓ |
| Orchard | Optional | ✓ | ✓ | ✓ |

Addressing the Drawbacks

1. 120 dB Siren Is Not Enough for Elephants

Do not depend on a high-volume omnidirectional siren. Instead:

- Use directional speakers aimed toward the intrusion point rather than broadcasting across the village.

- Combine with high-intensity directional LED floodlights.

- Trigger species-specific audio (e.g., bee-hive sounds or elephant distress calls, where supported by wildlife guidance).

- Notify the farmer and, for elephants, the Forest Department.

This reduces unnecessary disturbance while increasing the chance of changing the animal's path.

2. Village Residents Will Be Disturbed

Avoid a single loud alarm for the entire village.

Instead:

- Use localized directional audio only near the intrusion.

- Send mobile notifications to the farmer.

- Use lights as the first response.

- Escalate to stronger measures only if the animal remains.

3. Electromagnetic Waves Are Temporary

Do not rely on electromagnetic repellents as the primary solution.

Instead, use:

- AI vision

- Species-specific decision-making

- Directional lighting

- Localized audio

- Camera tracking

- Mobile alerts

These form a more reliable, layered system.

Farmer Node Sensor Setup with Battery Life

| Component | Purpose | Power Source | Estimated Battery Life* | Recharge Method |
| --- | --- | --- | --- | --- |
| 4 MP IR IP Camera | Captures images/videos for AI detection | Solar + 12 V Battery | 24–36 hours | Automatic solar charging during daytime |
| PIR Motion Sensor | Detects movement and wakes the camera | Solar + Battery | 7–10 days | Automatic solar charging |
| LDR Light Sensor | Detects day/night | Solar + Battery | 10+ days | Automatic solar charging |
| INA219 Battery Monitor | Monitors battery health | Solar + Battery | 10+ days | Automatic solar charging |
| ESP32 Controller | Controls sensors and communication | Solar + Battery | 48–72 hours | Automatic solar charging |
| Relay Module | Switches deterrent devices | Solar + Battery | 5–7 days | Automatic solar charging |
| Directional Speaker | Species-specific audio deterrent | Solar + Battery | 8–12 hours of active playback (used only during intrusions) | Automatic solar charging |
| LED Floodlight (100–150 W) | Night-time visual deterrent | Solar + Battery | 2–4 hours of continuous operation (normally activated for only 30–60 seconds per event) | Automatic solar charging |
| Complete Farmer Node | Entire monitoring unit | Solar + Battery | 24–48 hours without sunlight under normal operation | 50–100 W solar panel + charge controller |